📘 Common Ports and Services (CCNA 1 Reference Table)

| Port Number | Protocol | Service / Application | Description / Usage |
| --- | --- | --- | --- |
| 20 | TCP | FTP (Data) | File Transfer Protocol – data channel |
| 21 | TCP | FTP (Control) | File Transfer Protocol – control commands |
| 22 | TCP | SSH | Secure Shell – secure remote login |
| 23 | TCP | Telnet | Remote terminal access (unencrypted) |
| 25 | TCP | SMTP | Simple Mail Transfer Protocol – send email |
| 53 | TCP / UDP | DNS | Domain Name System – name resolution |
| 67 | UDP | DHCP (Server) | Dynamic Host Configuration Protocol – assigns IP addresses |
| 68 | UDP | DHCP (Client) | Receives IP address from DHCP server |
| 69 | UDP | TFTP | Trivial File Transfer Protocol – simple file transfer (used in Cisco IOS updates) |
| 80 | TCP | HTTP | Web browsing (unencrypted) |
| 110 | TCP | POP3 | Post Office Protocol – retrieves emails |
| 143 | TCP | IMAP | Internet Message Access Protocol – syncs emails |
| 161 | UDP | SNMP | Simple Network Management Protocol – monitoring and management |
| 162 | UDP | SNMP Trap | Receives SNMP alerts from network devices |
| 389 | TCP / UDP | LDAP | Lightweight Directory Access Protocol – directory services |
| 443 | TCP | HTTPS | Secure web traffic (SSL/TLS encrypted) |
| 445 | TCP | SMB | Server Message Block – file sharing (Windows) |
| 514 | UDP | Syslog | System logging protocol |
| 3389 | TCP | RDP | Remote Desktop Protocol – remote GUI access |
| 554 | TCP / UDP | RTSP | Real-Time Streaming Protocol – video/audio streaming |